SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 2 – PSEUDOCODE WRITING TASK (ALGORITHM DESIGN)

| Course Code | CSA1204 | Course Title | Computer Architecture for Multicore Systems |
| --- | --- | --- | --- |
| CO Assessed | CO5 | Assessment | Assessment Tool 2 – Pseudocode Writing Task |
| Weightage | 30% | Total Marks | 25 |
| Student Name | V.Samson | Reg. No. | 192511472 |
| Department | BE.CSE | Date | 27/08/2026 |

ANNEXURE A

Pseudocode Writing Task (Algorithm Design)

- Write pseudocode to design a DMA-based data transfer algorithm that moves data from an I/O device to memory while handling interrupts and minimizing CPU involvement.

- Develop pseudocode for an interrupt handling system that prioritizes vectored interrupts from multiple devices and dispatches appropriate service routines.

- Write pseudocode to select between memory-mapped I/O and I/O-mapped I/O dynamically based on latency and bandwidth requirements.

- Design pseudocode for a bus arbitration algorithm that manages multiple devices communicating over PCIe, USB, and Thunderbolt interfaces.

- Write pseudocode to implement a hybrid I/O communication mechanism that uses polling for low-speed devices and DMA with interrupts for high-speed devices.

Code of Conduct:

I  Am (V.Samson / 192511472) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student: V.Samson

MARKS ALLOCATION

|  | Problem Understanding (20%) | Algorithm Design (30%) | Use of Control Structures (20%) | Pseudocode Structure (10%) | Completeness (20%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 20% | Identifies all inputs, outputs, constraints accurately | Minor missing elements | Partial understanding | Incorrect interpretation |
| Algorithm Design | 30% | Complete, correct, and logically sound solution | Minor logical gaps | Partially correct logic | Incorrect approach |
| Use of Control Structures | 20% | Efficient and appropriate use of loops, conditions, functions/modules | Minor inefficiencies | Limited or basic usage | Incorrect or missing constructs |
| Pseudocode Structure | 10% | Well-structured, clear, consistent format, easy to follow | Mostly clear with minor issues | Difficult to follow in parts | Poorly structured and unclear |
| Completeness | 20% | Handles all cases; optimized approach | Handles most cases; reasonable efficiency | Handles basic cases only | Incomplete or inefficient |

1. DMA-Based Data Transfer from I/O Device to Memory

Problem understanding: The objective is to transfer a block of data from an I/O device to main memory using a Direct Memory Access (DMA) controller. The CPU should perform only initialization and completion handling, while the DMA controller performs the actual transfer. Interrupts are used to inform the CPU when the transfer completes or when an error occurs.

Inputs: I/O device identifier, source/device address, destination memory address, transfer size, DMA channel, transfer direction, and interrupt configuration.

Output: The requested data block is stored correctly in memory, with the CPU notified through an interrupt on completion or error.

Constraints/assumptions: The DMA controller and device support the requested transfer; the memory region is valid; the DMA channel is available; and the device can signal DMA completion/error.

Algorithm:

ALGORITHM DMA_Transfer_IO_To_Memory INPUT: device, deviceAddress, memoryAddress, transferSize, dmaChannel OUTPUT: transferStatus 1. CPU verifies that deviceAddress and memoryAddress are valid. 2. CPU verifies that transferSize > 0 and the DMA channel is available. 3. IF any validation fails THEN RETURN ERROR_INVALID_REQUEST END IF 4. CPU configures DMA channel: SET sourceAddress = deviceAddress SET destinationAddress = memoryAddress SET byteCount = transferSize SET direction = IO_TO_MEMORY SET interruptOnComplete = TRUE SET interruptOnError = TRUE 5. CPU clears any previous DMA status flags. 6. CPU enables the DMA channel. 7. CPU starts the I/O device DMA request. 8. CPU performs other work; DO NOT poll the transfer continuously. 9. WHEN DMA interrupt occurs DO READ DMA status IF status indicates ERROR THEN DISABLE DMA channel CLEAR error condition transferStatus = FAILURE NOTIFY CPU of DMA error ELSE IF status indicates COMPLETE THEN VERIFY transferred byte count = transferSize IF verification succeeds THEN transferStatus = SUCCESS ELSE transferStatus = FAILURE END IF DISABLE DMA channel RELEASE DMA channel END IF END WHEN 10. IF transferStatus = SUCCESS THEN RETURN SUCCESS ELSE RETURN FAILURE END IF END ALGORITHM

Why this satisfies the rubric: The algorithm minimizes CPU involvement by using DMA for the data movement and interrupts for notification. It includes validation, configuration, completion, error handling, verification, resource release, and conditional control structures, making the design complete rather than a basic transfer-only solution.

2. Prioritized Vectored Interrupt Handling for Multiple Devices

Problem understanding: Multiple hardware devices may generate interrupts at the same time. The system must identify the source using a vector, prioritize pending interrupts, and dispatch the correct Interrupt Service Routine (ISR). Higher-priority interrupts must be serviced first while lower-priority requests remain pending.

Inputs: Interrupt requests from multiple devices, interrupt vector table, device priority levels, CPU interrupt state, and ISR addresses.

Output: The appropriate ISR is executed for each pending interrupt, and interrupt status is restored correctly.

Constraints/assumptions: Every device has a valid interrupt vector and ISR; priority levels are known; and the interrupt controller can report pending requests and acknowledge them.

Algorithm:

ALGORITHM Prioritized_Vectored_Interrupt_Handler INPUT: interruptController, vectorTable, priorityTable OUTPUT: completedInterruptServices 1. SAVE current CPU context. 2. DISABLE interrupts of equal or lower priority during critical dispatch steps. 3. pendingSet = interruptController.GET_PENDING_INTERRUPTS() 4. WHILE pendingSet is not empty DO selectedInterrupt = NONE highestPriority = -1 FOR each interrupt in pendingSet DO priority = priorityTable[interrupt] IF priority > highestPriority THEN highestPriority = priority selectedInterrupt = interrupt END IF END FOR vectorNumber = interruptController.GET_VECTOR(selectedInterrupt) isrAddress = vectorTable[vectorNumber] IF isrAddress is invalid THEN RECORD invalid-vector error interruptController.CLEAR(selectedInterrupt) ELSE interruptController.ACKNOWLEDGE(selectedInterrupt) CALL ISR at isrAddress interruptController.CLEAR_OR_EOI(selectedInterrupt) END IF pendingSet = interruptController.GET_PENDING_INTERRUPTS() END WHILE 5. RESTORE saved CPU context. 6. RESTORE previous interrupt-enable state. 7. RETURN FROM INTERRUPT. END ALGORITHM

Why this satisfies the rubric: The design explicitly uses loops, conditions, a priority-selection mechanism, vector lookup, acknowledgement, ISR dispatch, invalid-vector handling, and repeated checking of pending requests. This ensures that multiple simultaneous interrupts are handled systematically and according to priority.

3. Dynamic Selection of Memory-Mapped I/O and I/O-Mapped I/O

Problem understanding: The system should select an I/O addressing technique dynamically according to latency and bandwidth requirements. Memory-mapped I/O provides a unified address space and can use normal memory instructions, while I/O-mapped I/O uses a separate I/O address space and dedicated I/O instructions on architectures that support it. The selection should consider both the requested performance and the hardware capability.

Inputs: Required latency, required bandwidth, operation type, device capabilities, CPU architecture capabilities, and current system load.

Output: Selected I/O mode and the corresponding communication procedure.

Constraints/assumptions: The architecture supports the selected mechanism; device registers are correctly mapped; and thresholds for latency/bandwidth are defined by the system designer.

Algorithm:

ALGORITHM Select_IO_Mapping INPUT: requiredLatency, requiredBandwidth, operationType, supportsMemoryMapped, supportsIOMapped, systemLoad OUTPUT: selectedMode 1. DEFINE LOW_LATENCY_THRESHOLD 2. DEFINE HIGH_BANDWIDTH_THRESHOLD 3. IF supportsMemoryMapped = FALSE AND supportsIOMapped = FALSE THEN RETURN ERROR_NO_SUPPORTED_IO_MODE END IF 4. IF requiredBandwidth >= HIGH_BANDWIDTH_THRESHOLD THEN IF supportsMemoryMapped = TRUE THEN selectedMode = MEMORY_MAPPED_IO ELSE selectedMode = IO_MAPPED_IO END IF 5. ELSE IF requiredLatency <= LOW_LATENCY_THRESHOLD THEN IF supportsMemoryMapped = TRUE THEN selectedMode = MEMORY_MAPPED_IO ELSE selectedMode = IO_MAPPED_IO END IF 6. ELSE IF supportsIOMapped = TRUE AND systemLoad is high THEN selectedMode = IO_MAPPED_IO ELSE IF supportsMemoryMapped = TRUE THEN selectedMode = MEMORY_MAPPED_IO ELSE selectedMode = IO_MAPPED_IO END IF END IF 7. IF operationType requires repeated register access THEN PREFER the mode with lower estimated access overhead END IF 8. CONFIGURE device using selectedMode. 9. RETURN selectedMode. END ALGORITHM

Design note: The thresholds are architecture- and workload-dependent; they should be measured or specified during system configuration rather than assumed to have one universal value. The algorithm therefore makes the selection policy explicit while checking hardware support before use.

4. Bus Arbitration for PCIe, USB, and Thunderbolt Devices

Problem understanding: Multiple devices may request communication resources at the same time. A bus arbitration mechanism must collect requests, consider interface capabilities and priority, select a requester, grant access, enforce fairness, and revoke or reassign the grant after completion or timeout. PCIe, USB, and Thunderbolt have different underlying protocols, so the arbitration layer should treat the interface as a resource while delegating protocol-specific transfer details to the corresponding interface controller.

Inputs: Active transfer requests, interface type (PCIe/USB/Thunderbolt), priority, requested bandwidth, request age, device state, and controller availability.

Output: A granted transfer opportunity for one eligible request at a time, followed by completion/failure handling and fair scheduling.

Constraints/assumptions: Each interface controller exposes request, grant, completion, and error status; maximum transfer time is bounded; and priority values are defined by system policy.

Algorithm:

ALGORITHM Multi_Interface_Bus_Arbitration INPUT: requestQueue, interfaceControllers OUTPUT: completedRequests 1. INITIALIZE ageCounter for every queued request. 2. REPEAT while the system is active 3. eligibleRequests = EMPTY 4. FOR each request in requestQueue DO 5. IF request.device is ready AND interfaceControllers[request.interface].isAvailable = TRUE THEN 6. ADD request to eligibleRequests 7. END IF 8. END FOR 9. IF eligibleRequests is empty THEN 10. WAIT for a request, completion, or controller event 11. CONTINUE 12. END IF 13. FOR each request in eligibleRequests DO 14. effectivePriority = request.priority + AGE_WEIGHT * ageCounter[request] 15. IF request is near its deadline THEN 16. effectivePriority = effectivePriority + DEADLINE_BONUS 17. END IF 18. END FOR 19. selectedRequest = request with maximum effectivePriority 20. controller = interfaceControllers[selectedRequest.interface] 21. controller.GRANT(selectedRequest.device) 22. controller.START_TRANSFER(selectedRequest) 23. startTime = CURRENT_TIME 24. WHILE transfer is not complete DO 25. IF controller.REPORTS_ERROR(selectedRequest) THEN 26. controller.ABORT(selectedRequest) 27. MARK selectedRequest as FAILED 28. BREAK 29. END IF 30. IF CURRENT_TIME - startTime > MAX_TRANSFER_TIME THEN 31. controller.ABORT(selectedRequest) 32. MARK selectedRequest as TIMEOUT 33. BREAK 34. END IF 35. END WHILE 36. IF transfer completed successfully THEN 37. MARK selectedRequest as COMPLETED 38. END IF 39. REMOVE completed/failed request from requestQueue 40. RESET ageCounter for selectedRequest 41. INCREASE ageCounter for waiting requests 42. controller.RELEASE(selectedRequest.device) 43. UNTIL system shutdown END ALGORITHM

Interface handling note: The arbitration policy is common, but the actual transfer command is delegated to the appropriate controller. PCIe requests are handled through the PCIe controller, USB transactions through the USB controller, and Thunderbolt traffic through the Thunderbolt controller. This avoids incorrectly treating these interfaces as identical buses while still providing a unified scheduling policy.

5. Hybrid I/O: Polling for Low-Speed Devices and DMA with Interrupts for High-Speed Devices

Problem understanding: A hybrid I/O mechanism should avoid wasting CPU time on high-speed transfers while also avoiding excessive interrupt overhead for very low-speed devices. Low-speed devices can be checked periodically by polling, whereas high-speed devices should use DMA and interrupts.

Inputs: Device list, device speed/classification, transfer size, device status, DMA availability, and polling interval.

Output: Data is transferred using the most suitable mechanism, with CPU overhead reduced and device-specific completion/error handling performed.

Constraints/assumptions: Low-speed devices can tolerate the polling interval; high-speed devices support DMA and interrupts; and the DMA controller has enough channels/resources.

Algorithm:

ALGORITHM Hybrid_IO_Communication INPUT: deviceList, DMAController, pollingInterval OUTPUT: transferResults 1. FOR each device in deviceList DO 2. IF device.speedClass = LOW_SPEED THEN 3. device.mode = POLLING 4. ELSE 5. device.mode = DMA_WITH_INTERRUPT 6. END IF 7. END FOR 8. START background polling service. 9. START interrupt service for DMA-capable devices. 10. REPEAT while system is active 11. FOR each device where device.mode = POLLING DO 12. IF CURRENT_TIME >= device.nextPollTime THEN 13. status = device.READ_STATUS() 14. IF status indicates DATA_READY THEN 15. data = device.READ_DATA() 16. STORE data in appropriate buffer 17. ELSE IF status indicates ERROR THEN 18. RECORD device error 19. ATTEMPT recovery if supported 20. END IF 21. device.nextPollTime = CURRENT_TIME + pollingInterval 22. END IF 23. END FOR 24. FOR each device where device.mode = DMA_WITH_INTERRUPT DO 25. IF device.hasPendingTransfer AND DMA channel is available THEN 26. CONFIGURE DMA for device buffer and transfer size 27. ENABLE DMA completion and error interrupts 28. START DMA transfer 29. MARK device transfer as IN_PROGRESS 30. END IF 31. END FOR 32. WHEN a DMA interrupt occurs DO 33. READ DMA status 34. IF status = COMPLETE THEN 35. VERIFY transfer count and buffer status 36. MARK transfer as SUCCESS 37. RELEASE DMA channel 38. ELSE IF status = ERROR THEN 39. RECORD DMA/device error 40. RELEASE or reset DMA channel as required 41. MARK transfer as FAILURE 42. END IF 43. END WHEN 44. SCHEDULE or perform other CPU work without busy-waiting on DMA transfers. 45. UNTIL system shutdown END ALGORITHM